| Vibration diagnostic report 3502-2021 |  |  |
| --- | --- | --- |
| Project: BW Panther IMO no: 9694464 Ordered by: BW Group | Date of measurement: 2021-09-09 | Place of measurement: During normal operation |

Measurement condition

Measurements were taken during normal operating condition.

Results presentation

Measured values are presented in the table below. Each machine if applicable is separated for driver (el. motor, diesel engine, etc.) and driven unit (pump, compressor, etc.). First column of the table consist name of the equipment. Second column contains the highest value of vibration velocity measured on the equipment in all measurement points. Third column contains classification of the vibration class according to proper ISO standard and other normative documents. Classification depends on highest reading of measured equipment only. Fourth column contains additional readings of enveloped value of acceleration, which is helpful in detection of early stage of bearing wear. Fifth column contains remarks and suggestions based on the analysis of vibration signal. This column can be taken as the final conclusion about machine condition. If cell is empty, it means that there is no existing problem or defect shown in vibration signal.

Vibration standards

Following standards may applied for assessment:

| ISO 10816-3 | Mechanical vibration — Evaluation of machine vibration by measurements on non-rotating parts — Part 3: Industrial machines with nominal power above 15 kW and nominal speeds between 120 r/min and 15 000 r/min when measured in situ |
| --- | --- |
| ISO 20816-1 | Mechanical vibration — Measurement and evaluation of machine vibration — Part 1: General guidelines |

Legend according to vibration class

| Cl. A | Newly commissioned |
| --- | --- |
| Cl. B | Unrestricted |
| Cl. C | Restricted long-term operation |
| Cl. D | High probability of damage, action required |
| Cl. D | Vibrations over the limits but actions are not required. |

Results
In table are presented only readings with max. RMS results for each device equipment:

| Machine name | Velocity RMS (mm/s) Max | ISO standard | Bearing Envelope 0-Peak (m/s2) Max | Remarks and suggestions |
| --- | --- | --- | --- | --- |
| Engine room |  |  |  |  |
| AE LO priming pumps |  |  |  |  |
| DG pre-lub pump no1 el. motor | 1,974 | Limit count error | - |  |
| FO circulation pumps |  |  |  |  |
| FO circulation pump no1 el. motor | 7,393 | Limit count error | - | Trend should be controlled. Actions to be taken based on trend result. |
| FO circulation pump no1 | 2,444 | Limit count error | - |  |
| Steering gear hydraulic unit pumps |  |  |  |  |
| Steering gear hydraulic unit pump no1 el. motor | 2,366 | Limit count error | - |  |
| Steering gear hydraulic unit pump no2 el. motor | 2,516 | Limit count error | - |  |

Measurement equipment:

| Technical data |  |
| --- | --- |
| Maker: | Info Marine |
| Type: | MarVib DC750 |
| Serial number: | 7506874 |
| Measuring range: | 2Hz-30kHz / RPM = 60-20000 |
| Indication error: | ± 0,5% |

Equipment is calibrated, certificate for verification - if required.

| Ship type: Oil/Chemical Tanker | Main dimensions: Length(b.p).......................................183,00 m Breadth(B.)........................................32,00 m |
| --- | --- |
| Sea depth: Least twice times greater than Vessel draught |  |
| Measurement method: According to standard ISO 10816 : - procedure No. 2 Measurement report |  |

Summary
Next measurements should be done in three month period to obtain trend value for each equipment, in some cases even one month period is preferable.

This report is prepared in good faith based on measurement diagnostic done on available running rotary machine and documentation submitted.

| Prepared by: | Approved by: |
| --- | --- |
| Service Engineer | Tomasz Chuchra |
| Bogusław Steciak | mob: 0048 600052257 |