Vibration Diagnostic Report

Report no: 7978-2026

| Project | COOLWATER |
| --- | --- |
| IMO no | 9377949 |
| Ordered by | Clearwater Ship Management |

| Data of measurement | 2026-01-30 - 2026-02-02 |
| --- | --- |
| Place of measurement | Antwerp - Vlissingen |
| Measurement condition | Measurements were taken during voyage from Antwerp to Vlissingen |

Approved by International Classification Societies

Results presentation

Measured values are presented in the table below. Each machine if applicable is separated for driver (el. motor, diesel engine, etc.) and driven unit (pump, compressor, etc.). First column of the table consist name of the equipment. Second column contains the highest value of vibration velocity measured on the equipment in all measurement points. Third column contains classification of the vibration class according to proper ISO standard and other normative documents. Classification depends on highest reading of measured equipment only. Fourth column contains additional readings of enveloped value of acceleration, which is helpful in detection of early stage of bearing wear. Fifth column contains vibration trend values if previous results are available from the same source. Sixth column contains remarks and suggestions based on the analysis of vibration signal. This column can be taken as the final conclusion about machine condition. If cell is empty, it means that there is no existing problem or defect shown in vibration signal.

Vibration standards

| ISO 10816-6 | Mechanical vibration - Evaluation of machine vibration by measurements on nonrotating parts - Part 6: Reciprocating machines with power ratings above 100 kW |
| --- | --- |
| ISO 10816-7 | Mechanical vibration — Evaluation of machine vibration by measurements on non-rotating parts — Part 7: Rotodynamic pumps for industrial applications, including measurements on rotating shafts |
| ISO 20283-4 | Mechanical vibration — Measurement of vibration on ships — Part 4: Measurement and evaluation of vibration of the ship propulsion machinery |
| ISO 20816-1 | Mechanical vibration — Measurement and evaluation of machine vibration — Part 1: General guidelines |
| ISO 20816-3 | Mechanical vibration — Measurement and evaluation of machine vibration — Part 3: Industrial machinery with a power rating above 15 kW and operating speeds between 120 r/min and 30 000 r/min |
| ISO 8528-9 | Reciprocating internal combustion engine driven alternating current generating sets — Part 9: Measurement and evaluation of mechanical vibrations |
| And makers recommendations |  |

Legend according to vibration class

Cl. A Newly commissioned

Cl. B Unrestricted

Cl. C Restricted long-term operation

Cl. D High probability of damage, action required

Cl. D Vibrations over the limits but actions are not required.

V. I Unrestricted

V. II Restricted long-term operation

V. III High probability of damage, action required

V. III Vibrations over the limits but actions are not required.

In limit Unrestricted

Out of limit High probability of damage, action required

Out of limit Vibrations over the limits but actions are not required.

Results

In table are presented only readings with max. RMS results for each device equipment:

| Machine name | Velocity RMS (mm/s) Max | ISO standard | Bearing Envelope 0-Peak (m/s2) Max | Trend Velocity RMS (mm/s) Max | Remarks and suggestions |
| --- | --- | --- | --- | --- | --- |
| Engine Room |  |  |  |  |  |
| Main Engine | 45.0(VSG) | Cl. C | - |  |  |
| Shaft alternator | 3.174 | V. I | 31.752 |  |  |
| Machine name | Velocity RMS (mm/s) Max | ISO standard | Bearing Envelope 0-Peak (m/s2) Max | Trend Velocity RMS (mm/s) Max | Remarks and suggestions |
| Main gearbox | 4.402 | Cl. B | 153.348 |  |  |
| Stern Tube | 0.844 | Cl. A | - |  |  |
| Auxiliary engines |  |  |  |  |  |
| Auxiliary Engine no1 | 34.2 | In limit | - |  |  |
| GO purifier no1 | 14.32 | Cl. D | - |  | High signal comes from the rotational speed of the purifier bowl. 1. Purifier should be checked/cleaned. |
| LO purifier no1 el. motor | 4.803 | Cl. D | 16.709 |  | High signal only in one point and comes from the operation of the purifier. No action is required for now. |
| LO purifier no1 | 5.108 | Cl. C | - |  |  |
| Compressors |  |  |  |  |  |
| Starting air compressor no1 el. motor | 9.068 | Cl. C | 71.407 |  |  |
| Starting air compressor no1 | 7.573 | Cl. C | 224.56 |  |  |
| Starting air compressor no2 el. motor | 22.666 | Cl. D | 87.314 |  | High signal comes from the rotation speed of the machine. 1. All bolts responsible for stiffness of structure should be checked/retightened. |
| Starting air compressor no2 | 13.681 | Cl. C | 201.469 |  |  |

Trend results:

Whenever new results are increased more than 5% of previous measurements

Whenever new results are in range plus / minus 5% of previous measurements

Whenever new results are reduced more than 5% of previous measurements

Measurement equipment:

| Technical data |  |
| --- | --- |
| Maker: | Info Marine |
| Type: | MarVib DC807 |
| Serial number: | RP303Z0981 |
| Measuring range: | up to 26kHz |
| Indication error: | 10% |

Equipment is calibrated, certificate for verification - if required.

| Ship type: Chemical/Oil Tanker | Main dimensions: Length(b.p).......................................99,98 m Breadth(B.)........................................16,00 m |
| --- | --- |
| Sea depth: Least twice times greater than Vessel draught |  |
| Measurement method: According to standard ISO 10816 : - procedure No. 2 Measurement report |  |

Summary

This report is prepared in good faith based on measurement diagnostic done on available running rotary machine and documentation submitted.

.

| Prepared by: |  | Approved by: |
| --- | --- | --- |
| Service Engineer |  | Nagoor Basha Kalluri |
| Devansh Bhardwaj |  | mob: 0091 9494030603 ISO CAT-IV Vibration analyst VA CAT-IV certificate no (M-168646-01) |