Reliability report

Project: APL Boston IMO no: 9597496 Ordered by: CMA CGM

Reliability Assessment Methodology

This report presents a predictive reliability analysis for machinery installed across sister vessels. Its primary objective is to support maintenance planning and operational risk management by estimating the probability of continued equipment performance over a six-month period.

The analysis calculates the reliability of individual machinery based on available condition monitoring data and compares these values with corresponding equipment on sister vessels. This comparative approach helps identify performance deviations, assess systemic risks, and prioritize maintenance actions across the fleet

Scope of Data

Each machinery entry includes:

- Machinery Name

- Date of Last Vibration Measurement

- Predicted Reliability Value – Probability of failure-free operation over the next six calendar months.

Reliability Calculation

Reliability is calculated using the exponential reliability function:

R(t)=e-λt

Where:

- R(t) is the reliability at time t,

- λ is the estimated failure rate (assumed constant),

- t is the time interval (here, t=182 days).

The failure rate λ is derived from historical maintenance records captured in the Condition-Based Maintenance system. Specifically, it reflects the frequency of corrective actions performed over time, providing a data-driven estimate of equipment degradation trends.

This model assumes a constant failure rate and is widely used when detailed failure distribution data is unavailable.

Time Basis

All reliability values are based on calendar time, not running hours, due to inconsistent runtime data availability across machinery. While this may introduce variability based on usage patterns, it ensures consistent comparison across the fleet.

Group-Level Reliability

In addition to individual machinery assessments, the report includes:

- Functional Group Reliability – Aggregated reliability values for machinery performing the same function across all sister vessels.

- This helps identify systemic performance trends and supports fleet-wide maintenance strategies.

- It also enables detection of machinery whose reliability significantly deviates from the group average, highlighting potential outliers that may require targeted investigation or corrective action.

Results

| Machine name | Date of last measurements | Reliability for next 6 months | Reliability of group for next 6 months |
| --- | --- | --- | --- |
| MAJOR MACHINERY |  |  |  |
| ME LO purifiers |  |  |  |
| ME LO purifier no1 el. motor | 2025-09-26 | 46.08% | 83.09% |
| ME LO purifier no1 | 2025-09-26 | 41.82% | 83.12% |
| ME LO purifier no2 el. motor | 2025-09-26 | 41.80% | 83.09% |
| ME LO purifier no2 | 2025-09-26 | 41.80% | 83.12% |
| AE LO purifiers |  |  |  |
| AE LO purifier no1 el. motor | 2025-09-26 | 82.39% | 87.38% |
| AE LO purifier no1 | 2025-09-26 | 61.61% | 87.39% |
| AE LO purifier no2 el. motor | 2025-09-26 | 90.76% | 87.38% |
| AE LO purifier no2 | 2025-09-26 | 90.76% | 87.39% |
| REST OF MACHINES |  |  |  |
| DG pre-lub pumps |  |  |  |
| DG1 pre-lub pump el. motor | 2025-10-27 | 90.85% | 90.74% |
| DG1 pre-lub pump | 2025-10-27 | 90.85% | 94.88% |
| DG2 pre-lub pump el. motor | 2025-10-27 | No maintenance in history | 90.74% |
| DG2 pre-lub pump | 2025-10-27 | No maintenance in history | 94.88% |
| DG3 pre-lub pump el. motor | 2025-09-26 | 75.03% | 90.74% |
| DG3 pre-lub pump | 2025-09-26 | 75.03% | 94.88% |
| DG4 pre-lub pump el. motor | 2025-10-27 | No maintenance in history | 90.74% |
| DG4 pre-lub pump | 2025-10-27 | 90.92% | 94.88% |
| FWG ejector pump |  |  |  |
| FW generator ejector pump el. motor | 2025-09-26 | 62.21% | 90.61% |
| FW generator ejector pump | 2025-10-27 | 90.94% | 96.47% |

| Ship type: Container Ship | Main dimensions: Length(b.p).......................................328,20 m Breadth(B.)........................................45,20 m |
| --- | --- |
| Sea depth: Least twice times greater than Vessel draught |  |
| Measurement method: According to standard ISO 10816 : - procedure No. 2 Measurement report |  |

Summary

This report is prepared in good faith based on measurement diagnostic done on available running rotary machine and documentation submitted.

| Prepared by: | Approved by: |
| --- | --- |
| Service Engineer | Tomasz Chuchra |
| Paulina Boroń | mob: 0048 600052257 |